OPC Unified Architecture

Specification

Part 13: Aggregates

Release Candidate

Version 1.00

July 15, 2008

Send comments to:
UAcomments@opcfoundation.org

| Specification Type | Industry Standard Specification |  |  |
| --- | --- | --- | --- |
| Title: | OPC Unified Architecture Aggregates | Date: | July 15, 2008 |
| Version: | Release Candidate 1.00 | Software | MS-Word |
|  |  | Source: | input.doc |
| Author: | OPC Foundation | Status: | Release Candidate |

CONTENTS

Page

FOREWORD	viii

AGREEMENT OF USE	viii

1	Scope	1

2	Reference documents	1

3	Terms, definitions, and abbreviations	1

3.1	OPC UA Part 1 terms	1

3.2	OPC UA Part 3 terms	1

3.3	OPC UA Part 4 terms	2

3.4	OPC UA Part 11 terms	2

3.5	OPC UA Aggregate terms	2

3.5.1	Aggregate	2

3.5.2	Interpolated	2

3.5.3	Extrapolated	2

3.5.4	SlopedInterpolation	2

3.5.5	SteppedInterpolation	2

3.6	Abbreviations and symbols	3

4	Concepts	4

4.1	General	4

4.2	History Objects	5

4.2.1	General	5

4.2.2	HistoryAggregateType	5

5	Aggregate specific usage of Services	7

5.1	General	7

5.2	Historical Nodes StatusCodes	7

5.2.1	Overview	7

5.2.2	Operation level result codes	7

5.2.3	Aggregate Information Bits	7

5.3	Aggregate Details	8

5.3.1	General	8

5.3.2	Common characteristics	8

5.3.3	Aggregate specific characteristics	11

Annex A Aggregate Specific Examples – Historical Access	18

A.1	Historical Aggregate specific characteristics	18

A.1.1	Example aggregate data – Historian 1	18

A.1.2	Example aggregate data – Historian 2	19

A.1.3	Example aggregate data – Historian 3	20

A.1.4	Example aggregate data – Historian 4	21

A.2	Interpolative	21

A.2.1	Description	21

A.2.2	Interpolated data with good bounding value.	21

A.2.3	Interpolated data with good bounding value with bad data in the interval.	22

A.2.4	Interpolated data with no good end bounding value.	22

A.2.5	Interpolated data with no good start bounding value.	23

A.3	Average	23

A.3.1	Description	23

A.3.2	Average data with good bounding value.	23

A.3.3	Average data with good bounding value with bad data in the interval.	24

A.3.4	Average data with no good end bounding value.	24

A.3.5	Average data with no good start bounding value.	25

A.4	TimeAverage	25

A.4.1	Description	25

A.4.2	TimeAverage data with good bounding value.	26

A.4.3	TimeAverage data with good bounding value  with bad data in the interval.	26

A.4.4	TimeAverage data with no good end bounding value.	27

A.4.5	TimeAverage data with no good start bounding value.	28

A.5	Total	29

A.5.1	Description	29

A.5.2	Total data with good bounding value.	29

A.5.3	Total data with good bounding value with bad data in the interval.	29

A.5.4	Total data with no good end bounding value.	30

A.6	TotalizeAverage	31

A.6.1	Description	31

A.6.2	TotalizeAverage data with good bounding value.	31

A.7	Minimum	31

A.7.1	Description	31

A.7.2	Minimum data with good bounding value.	31

A.7.3	Minimum data with good bounding value with bad data in the interval.	31

A.7.4	Minimum data with no good end bounding value.	32

A.7.5	Minimum data with no good start bounding value.	32

A.7.6	Minimum data with Partial Interval.	33

A.8	Maximum	33

A.8.1	Description	33

A.8.2	Maximum data with good bounding value.	33

A.8.3	Maximum data with good bounding value with bad data in the interval.	33

A.8.4	Maximum data with no good end bounding value.	34

A.8.5	Maximum data with no good start bounding value.	34

A.8.6	Maximum data with Partial Interval.	35

A.9	MininumActualTime	35

A.9.1	Description	35

A.9.2	MininumActualTime data with good bounding value.	35

A.9.3	MininumActualTime data with good bounding value with bad data in the interval.	35

A.9.4	MininumActualTime data with no good end bounding value.	36

A.9.5	MininumActualTime data with no good start bounding value.	36

A.9.6	MininumActualTime  with Partial Interval.	36

A.10	MaximumActualTime	37

A.10.1	Description	37

A.10.2	MaximumActualTime data with good bounding value.	37

A.10.3	MaximumActualTime data with good bounding value with bad data in the interval.	37

A.10.4	MaximumActualTime data with no good end bounding value.	37

A.10.5	MaximumActualTime data with no good start bounding value.	38

A.10.6	MaximumActualTime  with Partial Interval.	38

A.11	AnnotationCount	38

A.11.1	Description	38

A.11.2	Annotations on various data quality	39

A.12	Count	39

A.12.1	Description	39

A.12.2	Count data with good bounding value.	39

A.12.3	Count data with uncertain data in the interval.	39

A.13	DurationInState0	39

A.13.1	Description	39

A.13.2	Interpolated data with good bounding value.	39

A.13.3	Interpolated data with good bounding value with bad data in the interval.	40

A.13.4	Interpolated data with no good end bounding value.	40

A.13.5	Interpolated data with no good start bounding value.	40

A.14	DurationInState1	40

A.14.1	Description	40

A.14.2	Interpolated data with good bounding value.	40

A.14.3	Interpolated data with good bounding value with bad data in the interval.	41

A.14.4	Interpolated data with no good end bounding value.	41

A.14.5	Interpolated data with no good start bounding value.	41

A.15	NumberOfTransitions	41

A.15.1	Description	41

A.15.2	Interpolated data with good bounding value.	41

A.15.3	Interpolated data with good bounding value with bad data in the interval.	42

A.15.4	Interpolated data with no good end bounding value.	42

A.15.5	Interpolated data with no good start bounding value.	42

A.16	Start	42

A.16.1	Description	42

A.16.2	Start data with good bounding value.	42

A.16.3	Start data with bad bounding value.	43

A.16.4	Start data with good bounding value with bad data in the interval.	43

A.16.5	Start  data with partial intervals.	43

A.17	End	43

A.17.1	Description	43

A.17.2	End data with good bounding value.	44

A.17.3	End data with bad bounding value.	44

A.17.4	End data with good bounding value with bad data in the interval.	44

A.17.5	End  data with partial intervals.	44

A.18	DurationGood	45

A.18.1	Description	45

A.18.2	DurationGood data with good bounding value.	45

A.18.3	DurationGood data with good bounding value with bad data in the interval.	45

A.18.4	DurationGood data with no good end bounding value.	45

A.18.5	DurationGood data with no good start bounding value.	46

A.18.6	DurationGood data with uncertain data in the interval.	46

A.19	DurationBad	46

A.19.1	Description	46

A.19.2	DurationBad data with good bounding value.	46

A.19.3	DurationBad data with good bounding value with bad data in the interval.	47

A.19.4	DurationBad data with uncertain data in the interval.	47

Annex B Aggregate Specific Examples – Monitoried Item	48

B.1	Real-time Aggregate specific characteristics	48

B.1.1	Example aggregate data – Real-time Source 1	48

5.3.4	AggregateFilter	48

- Figures

Error! No table of figures entries found.

- Tables

Table 1 – Interpolation Examples	3

Table 2 – AggregateConfigurationType Definition	4

Table 3 – HistoryAggregateType Definition	5

Table 4 – Standard HistoryAggregateType BrowseNames	6

Table 5 – Uncertain operation level result codes	7

Table 6 – Data Location	7

Table 7 – Additional Information	8

Table 8 – History Aggregate Interval Information	9

Table 9 – Standard History Aggregate Data Type Information	10

Table 10 – AggregateFilter structure	48

Table 11 – AggregateFilterResult structure	49

OPC Foundation

____________

UNIFIED ARCHITECTURE –

FOREWORD

This specification is for developers of OPC UA clients and servers. The specification is a result of an analysis and design process to develop a standard interface to facilitate the development of servers and clients by multiple vendors that shall inter-operate seamlessly together.

Copyright © 2006, OPC Foundation, Inc.

AGREEMENT OF USE

COPYRIGHT RESTRICTIONS

Any unauthorized use of this specification may violate copyright laws, trademark laws, and communications regulations and statutes. This document contains information which is protected by copyright. All Rights Reserved. No part of this work covered by copyright herein may be reproduced or used in any form or by any means--graphic, electronic, or mechanical, including photocopying, recording, taping, or information storage and retrieval systems--without permission of the copyright owner.

OPC Foundation members and non-members are prohibited from copying and redistributing this specification. All copies must be obtained on an individual basis, directly from the OPC Foundation Web site
http://www.opcfoundation.org.

PATENTS

The attention of adopters is directed to the possibility that compliance with or adoption of OPC specifications may require use of an invention covered by patent rights. OPC shall not be responsible for identifying patents for which a license may be required by any OPC specification, or for conducting legal inquiries into the legal validity or scope of those patents that are brought to its attention. OPC specifications are prospective and advisory only. Prospective users are responsible for protecting themselves against liability for infringement of patents.

WARRANTY AND LIABILITY DISCLAIMERS

WHILE THIS PUBLICATION IS BELIEVED TO BE ACCURATE, IT IS PROVIDED "AS IS" AND MAY CONTAIN ERRORS OR MISPRINTS. THE OPC FOUDATION MAKES NO WARRANTY OF ANY KIND, EXPRESSED OR IMPLIED, WITH REGARD TO THIS PUBLICATION, INCLUDING BUT NOT LIMITED TO ANY WARRANTY OF TITLE OR OWNERSHIP, IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE OR USE. IN NO EVENT SHALL THE OPC FOUNDATION BE LIABLE FOR ERRORS CONTAINED HEREIN OR FOR DIRECT, INDIRECT, INCIDENTAL, SPECIAL, CONSEQUENTIAL, RELIANCE OR COVER DAMAGES, INCLUDING LOSS OF PROFITS, REVENUE, DATA OR USE, INCURRED BY ANY USER OR ANY THIRD PARTY IN CONNECTION WITH THE FURNISHING, PERFORMANCE, OR USE OF THIS MATERIAL, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

The entire risk as to the quality and performance of software developed using this specification is borne by you.

RESTRICTED RIGHTS LEGEND

This Specification is provided with Restricted Rights. Use, duplication or disclosure by the U.S. government is subject to restrictions as set forth in (a) this Agreement pursuant to DFARs 227.7202-3(a); (b) subparagraph (c)(1)(i) of the Rights in Technical Data and Computer Software clause at DFARs 252.227-7013; or (c) the Commercial Computer Software Restricted Rights clause at FAR 52.227-19 subdivision (c)(1) and (2), as applicable. Contractor / manufacturer are the OPC Foundation,. 16101 N. 82nd Street, Suite 3B, Scottsdale, AZ, 85260-1830

COMPLIANCE

The OPC Foundation shall at all times be the sole entity that may authorize developers, suppliers and sellers of hardware and software to use certification marks, trademarks or other special designations to indicate compliance with these materials. Products developed using this specification may claim compliance or conformance with this specification if and only if the software satisfactorily meets the certification requirements set by the OPC Foundation. Products that do not meet these requirements may claim only that the product was based on this specification and must not claim compliance or conformance with this specification.

Trademarks

Most computer and software brand names have trademarks or registered trademarks. The individual trademarks have not been listed here.

GENERAL PROVISIONS

Should any provision of this Agreement be held to be void, invalid, unenforceable or illegal by a court, the validity and enforceability of the other provisions shall not be affected thereby.

This Agreement shall be governed by and construed under the laws of the State of Minnesota, excluding its choice or law rules.

This Agreement embodies the entire understanding between the parties with respect to, and supersedes any prior understanding or agreement (oral or written) relating to, this specification.

ISSUE REPORTING

The OPC Foundation strives to maintain the highest quality standards for its published specifications, hence they undergo constant review and refinement. Readers are encouraged to report any issues and view any existing errata here:
http://www.opcfoundation.org/errata

# Scope

This specification is part of the overall OPC Unified Architecture specification series and defines the information model associated with Aggregates.

# Reference documents

[UA Part 1]	OPC UA Specification: Part 1 – Concepts, Version 1.0 or later

http://www.opcfoundation.org/UA/Part1/

[UA Part 3]	OPC UA Specification: Part 3 – Address Space Model, Version 1.0 or later

http://www.opcfoundation.org/UA/Part3/

[UA Part 4]	OPC UA Specification: Part 4 – Services, Version 1.0 or later

http://www.opcfoundation.org/UA/Part4/

[UA Part 5]	OPC UA Specification: Part 5 – Information Model, Version 1.0 or later

http://www.opcfoundation.org/UA/Part5/

[UA Part 7]	OPC UA Specification: Part 7 – Profiles, Version 1.0 or later

http://www.opcfoundation.org/UA/Part7/

[UA Part 8]	OPC UA Specification: Part 8 – Data Access, Version 1.0 or later

http://www.opcfoundation.org/UA/Part8/

[UA Part 9]	OPC UA Specification: Part 9 – Alarm & Conditions, Version 1.0 or later

http://www.opcfoundation.org/UA/Part9/

[UA Part 11]	OPC UA Specification: Part 11 – Historical Access, Version 1.0 or later

http://www.opcfoundation.org/UA/Par11/

# Terms, definitions, and abbreviations

## OPC UA Part 1 terms

The following terms defined in [UA Part 1] apply.

- Attribute

- BrowseName

- Event

- Node

- NodeId

- Object

- ObjectType

## OPC UA Part 3 terms

The following terms defined in [UA Part 3] apply.

- EventType

- Property

- Variable

## OPC UA Part 4 terms

- The following terms defined in [UA Part 4] apply.

- ExtensibleParameter

- StatusCode

## OPC UA Part 11 terms

- The following terms defined in [UA Part 11] apply.

- HistoricalNodes

## OPC UA Aggregate terms

### Aggregate

Provides summarized data values of process data.

An Aggregate is a way to produce a set of values derived from the raw data in the historian. Clients may specify an Aggregate when using the ReadHistory service. Complete details of the various standard Aggregates and their behaviour are outlined in Clause 5.3. Common Aggregates include averages over a given time range, minimum over a time range and maximum over a time range.

### Interpolated

Interpolated data is data that is calculated from the data in the archive. An interpolated value is calculated from the stored data points on either side of the requested timestamp.   The document refers to two types of interpolation, depending on how the historical data is represented.

### Extrapolated

Extrapolated data is data that is constructed from data outside of a discrete set of known data points. It is similar to the process of interpolation, which constructs new points between known points, but its results subject to greater uncertainty.  Extrapolated data is used in cases where the requested time period falls farther into the future than the data stored in the underlying system.

### SlopedInterpolation

SlopedInterpolation and/or Sloped Extrapolation refers to simple linear interpolation, as in the method of curve fitting using linear polynomials.

### SteppedInterpolation

SteppedInterpolation and/or Stepped Extrapolation refers to holding the last data point constant, or interpolating the value based on a horizontal line fit.

For example, consider the following table of raw and Interpolated/Extrapolated values:

Table 1 – Interpolation Examples

| Timestamp | Raw Value | Sloped Interpolation | Stepped Interpolation |
| --- | --- | --- | --- |
| 12:00:00 | 10 |  |  |
| 12:00:05 |  | 15 | 10 |
| 12:00:08 |  | 18 | 10 |
| 12:00:10 | 20 |  |  |
| 12:00:15 |  | 25 | 20 |
| 12:00:20 | 30 |  |  |
|  |  | Sloped Extrapolation | Stepped Extrapolation |
| 12:00:25 |  | 35 | 30 |
| 12:00:27 |  | 37 | 30 |

## Abbreviations and symbols

DA	Data Access

HDA	Historical Data Access

UA 	Unified Architecture

# Concepts

## General

The OPC UA Aggregates specification defines the representation of aggregated historical data and event data in the OPC Unified Architecture.  Included is the specification is the definition of Aggregates used in processed data retrieval.

#### AggregateConfigurationType

The AggregateConfigurationType defines the general characteristics of a node that defines the historical configuration of any variable or property that is defined to contain history. It is formally defined in Table 2.

Table 2 – AggregateConfigurationType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | AggregateConfigurationType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | NodeClass | BrowseName | DataType | TypeDefinition | ModellingRule |
| Subtype of the BaseObjectType defined in [UA Part 5] |  |  |  |  |  |
| HasProperty | Variable | TreatUncertainAsBad | Boolean | PropertyType | Mandatory |
| HasProperty | Variable | PercentDataBad | UInt8 | PropertyType | Mandatory |
| HasProperty | Variable | PercentDataGood | UInt8 | PropertyType | Mandatory |
| HasProperty | Variable | SteppedSlopedExtrapolation | Boolean | PropertyType | Mandatory |

The TreatUncertainAsBad Variable indicates how the server treats data returned with a StatusCode severity Uncertain with respect to Aggregate calculations.   A value of True indicates the server considers the severity equivalent to Bad, a value of False indicates the server considers the severity equivalent to Good.  The default value is True.

The PercentDataBad Variable indicates the Maximum percentage of bad data in a given  interval above which would cause the StatusCode for the given interval for processed data request to be set to Bad. (Uncertain is treated as defined above).  For values equal to or below this percentage the StatusCode would be Uncertain or Good. For details on which Aggregates use the PercentDataBad Variable, see the definition of each Aggregate. The default value is 0.

The PercentDataGood Variable indicates the minimum percentage of Good data in a given interval which would cause the StatusCode for the given interval for the processed data requests to be set to Good. For values below this percentage the StatusCode would be Uncertain or Bad. For details on which Aggregates use the PercentDataGood Variable, see the definition of each Aggregate. The default value is 100.

The SteppedSlopedExtrapolation Variable indicates how the server interpolates data when no boundary value exists (i.e. extrapolating into the future from the last known value).  A value of False indicates that the server will use a SteppedExtrapolation format, and hold the last known value constant.  A value of True indicates the server will project the value using Slopedinterpolation mode.  The default value is False.

The Stepped Variable specifies whether the historical data was collected in such a manner that it should be displayed as sloped interpolation (sloped Lines between point) or as SteppedInterpolation (vertically-connected horizontal lines between points) when raw data is examined.  This property also effect how some Aggregates are calculated.  A value of True indicates stepped interpolation mode.  A value of False indicates SlopedInterpolated mode.  The default value is False.

## History Objects

### General

OPC UA servers can support several different functionalities and capabilities.  The following standard Objects are used to expose these capabilities in a common fashion, and there are several standard defined concepts that can be extended by vendors.

### HistoryAggregateType

This ObjectType defines an Aggregate supported by a UA server. This object is formally defined in Table 3.

Table 3 – HistoryAggregateType Definition

| Attribute |  | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BrowseName |  | HistoryAggregateType |  |  |  |  |
| IsAbstract |  | False |  |  |  |  |
| References | Node Class |  | BrowseName | DataType | Type Definition | Mod. Rule |
| Subtype of the BaseObjectType defined in [UA Part 5] |  |  |  |  |  |  |

For the HistoryAggregateType, the Description Attribute (inherited from the Base NodeClass), is mandatory.  The Description Attribute provides a localized description of the Aggregate

Table 4 outlines the BrowseName and Description for the standard Aggregates.

Table 4 – Standard HistoryAggregateType BrowseNames

| BrowseName | Description |
| --- | --- |
|  | Interpolation Aggregate |
| Interpolative | Do not retrieve an aggregate. This is used for retrieving interpolated Values. |
|  | Data Averaging and Summation Aggregates |
| Average | Retrieve the average data over the resample interval. |
| TimeAverage | Retrieve the time weighted average data over the resample interval. |
| Total | Retrieve the sum of the data over the resample interval. |
| TotalizeAverage | Retrieve the totalized Value (time integral) of the data over the resample interval. |
|  | Data Variation Aggregates |
| Minimum | Retrieve the minimum Value in the resample interval. |
| Maximum | Retrieve the maximum Value in the resample interval. |
| MinimumActualTime | Retrieve the minimum value in the resample interval and the Timestamp of the minimum value. |
| MaximumActualTime | Retrieve the maximum value in the resample interval and the Timestamp of the maximum value. |
| Range | Retrieve the difference between the minimum and maximum Value over the sample interval. |
|  | Counting Aggregates |
| AnnotationCount | Retrieve the number of Annotations in the interval. |
| Count | Retrieve the number of raw Values over the resample interval. |
| DurationInState0 | Retrieve the time a Boolean was in a 0 state |
| DurationInState1 | Retrieve the time a Boolean was in a 1 state |
| NumberOfTransitions | Retrieve the number of state changes a Boolean value experianced in the interval |
|  | Time Aggregates |
| Start | Retrieve the Value at the beginning of the resample interval. The time stamp is the time stamp of the beginning of the interval. |
| End | Retrieve the Value at the end of the resample interval. The time stamp is the time stamp of the end of the interval. |
| Delta | Retrieve the difference between the first and last Value in the resample interval. |
|  | Data Quality Aggragates |
| DurationGood | Retrieve the duration of time in the interval during which the data is good. |
| DurationBad | Retrieve the duration of time in the interval during which the data is bad. |
| PercentGood | Retrieve the percent of data (0 to 100) in the interval which has good StatusCode. |
| PercentBad | Retrieve the percent of data (0 to 100) in the interval which has bad StatusCode. |
| WorstQuality | Retrieve the worst StatusCode of data in the interval. |

# Aggregate specific usage of Services

## General

[UA Part 4] specifies all Services needed for OPC UA Aggregates. In particular:

- The Browse Service Set or Query Service Set to detect Aggregates and their configuration.

- The HistoryRead Service of the Attribute Service Set  to read aggregated history of HistoricalNodes.

## Historical Nodes StatusCodes

### Overview

This section defines additional codes and rules that apply to the StatusCode when used for HistoricalNodes.

The general structure of the StatusCode is specified in [UA Part 4].  It includes a set of common operational result codes which also apply to historical data and/or events.

### Operation level result codes

In OPC UA Aggregates the StatusCode is used to indicate the conditions under which a Value or Event was stored, and thereby can be used as an indicator it’s usability. Due to the nature of aggregated data and/or events, additional information beyond the basic quality and call result code needs to be conveyed to the client.  For example, whether or not the result was Interpolated, were all data inputs to a calculation of good quality, etc.

Table 5 contains codes with Uncertain severity indicating that the value has been retrieved under sub-normal conditions. It is Important to note, that these are the codes that are specific for OPC UA Aggregates and supplement the codes that apply to all types of data and are therefore defined in [UA Part 4] , [UA Part 8] and [UA Part 11]

Table 5 – Uncertain operation level result codes

| Symbolic Id | Description |
| --- | --- |
| Uncertain_DataSubNormal | The value is derived from multiple values and has less than the required number of Good values. |

### Aggregate Information Bits

These bits are set only when reading historical data of HistoricalDataNodes. They indicate where the data value came from and provide information that affects how the client uses the data value. Table 6 lists the bit settings which indicate the data location (i.e. is the value stored in the underlying data repository, or is the value the result of data aggregation).  These bits are mutually exclusive.

Table 6 – Data Location

| StatusCode | Description |
| --- | --- |
| Raw | A raw data value. |
| Calculated | A data value which was calculated. |
| Interpolated | A data value which was interpolated. |

In the case where Interpolated data is requested, and there is an actual raw value for that timestamp, the server should set the ‘Raw’ bit in the StatusCode of that value.

Table 7 lists the bit settings which indicate additional important information about the data values returned.

Table 7 – Additional Information

| StatusCode | Description |
| --- | --- |
| Partial | A data value which was calculated with an incomplete interval. |
| Extra Data | A raw data value that hides other data at the same timestamp. |
| Multiple Values | Multiple values match the aggregate criteria (i.e. multiple minimum values or multiple worst quality at different timestamps within the same interval) |

The conditions under which these information bits are set depend on how the historical data has been requested and state of the underlying data repository.

## Aggregate Details

### General

The purpose of this section is to detail the requirements and behavior for OPC UA Servers supporting Aggregates. The intent is to standardize the Aggregates users can reliably predict the results of an Aggregate computation and understand its meaning. If users require custom functionality in the Aggregates, those Aggregates should be written as custom vendor defined Aggregates.

The standard Aggregates must be as consistent as possible, meaning that each Aggregate’s behavior must be similar to every other Aggregate’s behavior where input parameters, raw data, and boundary conditions are similar. Where possible, the Aggregates should deal with input and preconditions in a similar manner.

This section is divided up into two parts. The first sub section deals with Aggregate characteristics and behavior that are common to all Aggregates. The remaining sub sections deal with the characteristics and behavior of Aggregates that are aggregate-specific.

### Common characteristics

#### Description

This subsection deals with Aggregate characteristics and behavior that are common to all Aggregates.

#### Generating intervals

To read Aggregates, OPC clients must specify three time parameters:

-	start time (Start)

-	end time (End)

-	resample interval (Int)

The OPC server must use these three parameters to generate a sequence of time intervals and then calculate an Aggregate for each interval.  This section specifies, given the three parameters, which time intervals are generated.  Table 8 provides information on the intervals for each Start and End time combination.  Range is defined to be |End - Start|.

All interval Aggregates return a timestamp of the start of the interval unless otherwise noted for the particular Aggregate.

Table 8 – History Aggregate Interval Information

| Start/End Time | Resample Interval | Resulting Intervals |
| --- | --- | --- |
| Start = End | Int = Anything | No intervals. Returns a Bad_InvalidArgument StatusCode, regardless of whether there is data at the specified time or not. |
| Start < End | Int = 0 or Int ≥ Range | One interval, starting at Start and ending at End. Includes Start, excludes End, i.e., [Start, End). |
| Start < End | Int ≠ 0, Int < Range, Int divides Range evenly. | Range/Int intervals. Intervals are [Start, Start + Int), [Start + Int, Start + 2 * Int),..., [End - Int, End). |
| Start < End | Int ≠ 0, Int < Range, Int does not divide Range evenly. | Range/Int intervals. Intervals are [Start, Start + Int), [Start + Int, Start + 2 * Int),..., [Start + ( Range/Int - 1)* Int, Start + Range/Int * Int), [Start + Range/Int * Int, End). In other words, the last interval contains the “rest” that remains in the range after taking away Range/Int intervals of size Int. |
| Start > End | Int = 0 or Int ≥ Range | One interval, starting at Start and ending at End. Includes Start, excludes End, i.e., [End, Start). |
| Start > End | Int ≠ 0, Int < Range, Int divides Range evenly. | Range/Int intervals. Intervals are [Start - Int, Start], [Start – 2 * Int, Start – Int),..., [End, End + Int). |
| Start > End | Int ≠ 0, Int < Range, Int does not divide Range evenly. | Range/Int intervals. Intervals are [Start - Int, Start), [Start – 2 * Int, Start - Int),..., [Start - Range/Int * Int, Start – ( Range/Int - 1)* Int), [End, Start - Range/Int * Int). In other words, the last interval contains the “rest” that remains in the range after taking away Range/Int intervals of size Int starting at Start. |

#### Data types

Table 4 outlines the valid data types for each Aggregate.   Some Aggregates are intended for numeric data types – i.e. integers or real/floating point numbers. Dates, strings, arrays, etc. are not supported.   Other Aggregates are intended for digital data types – i.e. Boolean or enumerations.   In addition some Aggregates may return results with a different datatype than those used to calculate the aggregate.  Table 4 also outlines the default data type returned for each aggregate.

Table 9 – Standard History Aggregate Data Type Information

| BrowseName | Valid Data Type | Default Result Data Type |
| --- | --- | --- |
|  | Interpolation Aggregate |  |
| Interpolative | Numeric | Double |
|  | Data Averaging Aggregates |  |
| Average | Numeric | Double |
| TimeAverage | Numeric | Double |
| Total | Numeric | Double |
| TotalizeAverage | Numeric | Double |
|  | Data Variation Aggregates |  |
| Minimum | Numeric | Raw data type |
| Maximum | Numeric | Raw data type |
| MinimumActualTime | Numeric | Raw data type |
| MaximumActualTime | Numeric | Raw data type |
| Range | Numeric | Raw data type |
|  | Counting Aggregates |  |
| AnnotationCount | All | Integer |
| Count | All | Integer |
| DurationInState0 | Boolean | Duration |
| DurationInState1 | Boolean | Duration |
| NumberOfTransitions | Boolean | Integer |
|  | Time Aggregates |  |
| Start | All | Raw data type |
| End | All | Raw data type |
| Delta | Numeric | Raw data type |
|  | Data Quality Aggregates |  |
| DurationGood | All | Duration |
| DurationBad | All | Duration |
| PercentGood | All | Double |
| PercentBad | All | Double |
| WorstQuality | Numeric | StatusCode |

#### StatusCode calculation

For Aggregate values, the StatusCode for each returned Aggregate shall be Good, if the StatusCode for ALL values used in the Aggregate was Good.

If the StatusCode of ANY value used in computing the Aggregate was not Good, then the server must use the TreatUncertainAsBad, PercentDataBad and PercentDataGood parameter (see Clause 4.1.1.1) settings to determine the StatusCode of the resulting Aggregate for the interval.    Some Aggregates may explicitly define there own method of determining quality.

If the percentage of Good values in an interval is greater then or equal to the PercentDataGood, the Aggregate is considered Good.

If the percentage of Bad values in an interval is greater then or equal to the PercentDataBad, the Aggregate is considered Bad.

Since a value can be either Good or Bad only (Uncertain is defined as Good or Bad as per TreatUncertainAsBad  setting), percentage good = 100 – percentage bad.  If a percentage good (X) is in the following range Percentage bad < X < Percentage Good then the quality of the Aggregate is Uncertain_DataSubNormal.

### Aggregate specific characteristics

#### Description

This sub section deals with aggregate specific characteristics and behavior that is specific to a particular Aggregate.

#### Interpolative

In order for the interpolative Aggregate to return meaningful data, there must be good values at the boundary conditions.  For the purposes of discussion we will use the terms good and non-good.  As discussed in the StatusCode section (See Clause 5.3.2.4), what is represented by non-good is Server dependant.  For some Servers non-good represents only Bad data, for others it represents Bad and Uncertain data depending on the TreatUncertainAsBad setting.

When a non-good value is encountered at a boundary condition, the following rules must be followed:

- If the value at the requested time is non-good, the Aggregate looks for good data on both sides of the requested time in order to perform interpolation.

- The method of Interpolation, either SlopedInterpolation or SteppedInterpolation as determined by the Stepped attribute.   See Clause 4.1.1.1

- If there is no end bound (i.e. future time)  the value should be Extrapolated forward in time from the previous good value.  The method of Extrapolation, SlopedExtrapolation or SteppedExtrapolation will be server dependant. This is indicated by the SteppedSlopedExtrapolation property.   See Clause 4.1.1.1.

- The Aggregate should not Extrapolate backwards in time.  If there is no beginning bound, it should return Bad_NoData. The trailing value should not be pulled backward in time.  i.e. The last value should not be used with a modified timestamp of the beginning of the interval.

- If there happens to be a good raw value at the requested time, the raw value is returned.

- If any non-good values are skipped in order to find the closest good value, the Aggregate will be Uncertain_DataSubNormal

- Unless otherwise indicated, StatusCodes are Good, Interpolated.

#### Average

The average Aggregate adds up the values of all good raw data for each interval, and divides the sum by the number of good values. If any non-good values are ignored in the computation, the Aggregate StatusCode will be determined using the StatusCode Calculation (See Clause 5.3.2.4)

If no data exists for an interval, the StatusCode of the Aggregate for that interval will be Good_NoData.

All interval Aggregates return timestamp of the start of the interval. Unless otherwise indicated, StatusCodes are Good, Calculated.

The value of the Stepped attribute does not affect how the Aggregate is determined.

#### TimeAverage

The time weighted average Aggregate uses interpolation as described in the Interpolated section above to find the value of a point at the beginning and end of an interval. A straight line is drawn between each raw value in the interval. The area under the line is divided by the length of the interval to yield the average.  The time resolution used in this calculation is server specific, but it is recommended to be the standard time resolution used in the OPC UA specification (see definition of UtcTime in [UA Part 3])

The following examples are based on the sample Historian data in Annex A

For Example using Historian 1:

Given:

Start:  Jan-01-2002 12:00:10 End:  Jan-01-2002 12:00:15  Interval: 00:00:05

Then:

Point1 = Good Raw value of 10 at 12:00:10

Point2 =  slope Interpolated value of 15 at 12:00:15, using bounding values at 12:00:10 and 12:00:20.

Area under the line is 62500  (1/2 base*height + base*height).  Interval is 5000 milliseconds

TimeAverage = Area/interval = 12.5

For Example using Historian 3:

Given:

Start:  Jan-01-2002 12:00:10 End:  Jan-01-2002 12:00:15  Interval: 00:00:05

Then:

Point1 = Good Raw value of 10 at 12:00:10

Point2 =  step Interpolated value of 10 at 12:00:15, using bounding values from 12:00:

Area under the line is 50000  (base*height).  Interval is 5000 milliseconds

TimeAverage = Area/interval = 10

If any of an interval’s raw values are non-good, they are ignored, and the Aggregate StatusCode for that interval is determined using the StatusCode Calculation (See Clause 5.3.2.4)

#### Total

The total Aggregate adds up all the values of all good raw values for each interval.  If any non-good values are ignored in the computation, the Aggregate StatusCode will be determined using the StatusCode Calculation (See Clause 5.3.2.4).

If no data exists for an interval, the StatusCode of the Aggregate for that interval will be Good_NoData.

Unless otherwise indicated, StatusCodes are Good, Calculated

The value of the Stepped attribute does not affect how the Aggregate is determined.

#### TotalizeAverage

The TotalizeAverage Aggregate performs the following calculation for each interval:

TotalizeAverage = time_weighted_avg * interval_length (milliseconds)

Where:

Time_weighted_avg is the result from the TimeAverage Aggregate, using the interval supplied to the TotalizeAverage call.

Interval_length is the interval of the Aggregate.

The resulting units would be normalized to milliseconds, i.e. [time_weighted_avg Units]*milliseconds.

If any non-good values are ignored in the computation of an interval, the Aggregate StatusCode will be  determined using the StatusCode Calculation (See Clause 5.3.2.4).

All interval Aggregates return timestamp of the start of the interval. Unless otherwise indicated, StatusCodes are Good, Calculated

#### Minimum

The minimum Aggregate is the same as the minimum actual time, except the timestamp of the Aggregate will always be the start of the interval for every interval.

Unless otherwise indicated, StatusCodes are Good, Calculated.

The value of the Stepped attribute does not affect how the Aggregate is determined.

#### Maximum

This Aggregate is the same as the minimum, except the value is the maximum raw value within the interval [s,e).

Unless otherwise indicated, StatusCodes are Good, Calculated.

The value of the Stepped attribute does not affect how the Aggregate is determined.

#### MininumActualTime

The minimum actual time Aggregate retrieves the minimum good raw value within the interval [s,e), and returns that value with the timestamp at which that value occurs. Note that if the same minimum exists at more than one timestamp, the oldest one is retrieved, and the StatusCode is set to MultiValues.  If a non-good value is lower than the good minimum, the StatusCode of the Aggregate will be determined using the StatusCode Calculation (See Clause 5.3.2.4).

Unless otherwise indicated, StatusCodes are Good,Raw.   If no values are in the interval no data is returned with a timestamp of the start of the interval. If only bad quality values are available then the status is returned as Bad, Raw, The value is indeterminate, since some system may save bad values, but other may not.

The value of the Stepped attribute does not affect how the Aggregate is determined.

#### MaximumActualTime

This is the same as the minimum actual time Aggregate, except that the value is the maximum raw value within the interval [s,e). Note that if the same maximum exists at more than one timestamp, the oldest one is retrieved, and the StatusCode is set to MultiValues

Unless otherwise indicated, StatusCodes are Good, Raw.

The value of the Stepped attribute does not affect how the Aggregate is determined.

#### Range

The range Aggregate finds the difference between the raw maximum and raw minimum values in the interval. If only one value exists in the interval, the range is zero. Note that the range is always zero or positive.

If Stepped = True, Range is the difference between the SteppedInterpolated maximum and minimum values in the interval.

If there are any non-good raw values in the interval, they are ignored, and the Aggregate StatusCode will be Uncertain_DataSubNormal.

All interval Aggregates are returned with timestamp of the start of the interval. Unless otherwise indicated, StatusCodes are Good, Calculated.

#### AnnotationCount

This Aggregate returns a count of all Annotations.

The value of the Stepped attribute does not affect how the Aggregate is determined.

Unless otherwise indicated, StatusCodes are Good, Calculated

#### Count

This Aggregate retrieves a count of all the raw values within an interval. If one or more raw values are non-good, they are not included in the count, and the Aggregate StatusCode is  determined using the StatusCode Calculation (See Clause 5.3.2.4).  If no good data exists for an interval, the count is zero.

The value of the Stepped attribute does not affect how the Aggregate is determined.  That means that all good values are counted, even if multiple similar values exist at the same ‘step’.

Unless otherwise indicated, StatusCodes are Good, Calculated

#### DurationInState0

This Aggregate returns the time Duration during the resample interval that the variable was in the zero state. If one or more raw values are non-good, they are not included in the Duration, and the Aggregate StatusCode is determined using the StatusCode Calculation (See Clause 5.3.2.4).  If no good data exists for an interval, the Duration is 0.

Duration is in milliseconds.  Unless otherwise indicated, StatusCodes are Good, Calculated

#### DurationInState1

This Aggregate returns the time Duration during the resample interval that the variable was in the one state. If one or more raw values are non-good, they are not included in the Duration, and the Aggregate StatusCode is determined using the StatusCode Calculation (See Clause 5.3.2.4).  If no good data exists for an interval, the Duration is 0.

Duration is in milliseconds.  Unless otherwise indicated, StatusCodes are Good, Calculated

#### NumberOfTransitions

This Aggregate returns a count of the number of transition the variable had during the resample interval.  If one or more raw values are non-good, they are not included in the count, and the Aggregate StatusCode is determined using the StatusCode Calculation (See Clause 5.3.2.4).  If no good data exists for an interval, the number of transistions is 0.

Unless otherwise indicated, StatusCodes are Good, Calculated

#### Start

The start Aggregate retrieves the first raw value within the interval [s,e), and returns that value with the timestamp at which that value occurs.   If the value is non-good , then the StatusCode of the Aggregate will be Uncertain_DataSubNormal. Unless otherwise indicated, StatusCodes are Good, Raw.

Aggregate

The value of the Stepped attribute does not affect how the Aggregate is determined.

#### End

The end Aggregate retrieves the last raw value within the interval [s,e), and returns that value with the timestamp at which that value occurs.   If the value is non-good , than the StatusCode of the Aggregate will be Uncertain_DataSubNormal.

Unless otherwise indicated, StatusCodes are Good, Raw.

If Stepped = True and no raw value is in the interval, the Aggregate returns the SteppedInterpolated value at the beginning of the interval.

#### Delta

The delta Aggregate retrieves the difference between the earliest and latest good raw values in an interval. If the last value is less than the first value, the result will be negative. If the last value is the same as the first value, or if the last value is also the first value at the same timestamp, the result will be zero. If the last value is greater than the first value, the result will be positive.

Aggregate

The value of the Stepped attribute does not affect how the Aggregate is determined.

If any non-good values exist earlier or later than the earliest and latest good values, respectively, the Aggregate is Uncertain_DataSubNormal.

All interval Aggregates are returned with timestamp of the start of the interval. Unless otherwise indicated, StatusCodes are Good, Calculated.

#### DurationGood

The Duration good Aggregate looks at the StatusCode of a bounding value of the interval to determine what the StatusCode is at the beginning of the interval. If no bounding value exists, the StatusCode is assumed to be bad at the start of the interval.   This Aggregate only considers truly Good values.  Uncertain values are not considered Good for purposes of calculating this Aggregate.

Whenever a raw value x with quality q is encountered from beginning to end within an interval, the quality is considered to be q until the next value, y, is encountered, at which point the quality becomes that of y, and so on.

The time Duration is returned in milliseconds. No returned value will ever be Uncertain_DataSubNormal.

Each interval’s Aggregate is returned with timestamp of the start of the interval. StatusCodes are Good, Calculated

#### DurationBad

The Duration bad Aggregate looks at the quality of a bounding value of the interval to determine what the quality is at the beginning of the interval. If no bounding value exists, the quality is assumed to be bad at the start of the interval.   This Aggregate only considers truly Bad values.  Uncertain values are not considered bad for purposes of calculating this Aggregate.

Whenever a raw value x with quality q is encountered from beginning to end within an interval, the quality is considered to be q until the next value, y, is encountered, at which point the quality becomes that of y, and so on.

The time Duration is returned in milliseconds. No returned value will ever be uncertain or subnormal.

Each interval’s Aggregate is returned with timestamp of the start of the interval. StatusCodes are Good, Calculated.

Duration Bad is not simply the interval minus duration good, since the interval may contain uncertain data.

#### PercentGood

This Aggregate performs the following calculation:

percent_good = duration_good / interval_length * 100

Where:

duration_good is the result from the DURATIONGOOD Aggregate, calculated using the interval supplied to PERCENTGOOD call.

Interval_length is the interval of the Aggregates.

No returned value will ever be uncertain or subnormal.

Each interval’s Aggregate is returned with timestamp of the start of the interval. StatusCodes are Good, Calculated.

The interval_length is the entire sample interval, regardless of quality.

#### PercentBad

This Aggregate performs the following calculation:

percent_bad = duration_bad / interval_length * 100

Where:

duration_good is the result from the DURATIONBAD Aggregate, calculated using the interval supplied to PERCENTBAD call.

Interval_length is the interval of the Aggregates.

No returned value will ever be uncertain or subnormal.

Each interval’s Aggregate is returned with timestamp of the start of the interval. StatusCodes are Good, Calculated.

The interval_length is the entire sample interval, regardless of quality.

#### WorstQuality

This Aggregate returns the worst quality of the raw values in the interval. That is, Bad status are worse than Uncertain, which are worse than Good. No distinction is made between the specific reasons for the status.

This Aggregate returns the worst StatusCode as the value of the Aggregate.

The timestamp is always the start of the interval. The StatusCodes are Good, Calculated.

- Aggregate Specific Examples – Historical Access

  - Historical Aggregate specific characteristics

    - Example aggregate data – Historian 1

For the purposes of Historian 1 examples consider a source historian with the following data:

| Timestamp | Value | StatusCode | Notes |
| --- | --- | --- | --- |
| Jan-01-2002 12:00:00 | - | Bad_NoData | First archive entry, Point Created |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:20 | 20 | Raw, Good |  |
| Jan-01-2002 12:00:30 | 30 | Raw, Good |  |
| Jan-01-2002 12:00:40 | 40 | Raw, Bad | ANNOTATION: Jan-02-2002 8:00:00 Scan failed, Bad Data entered ANNOTATION: Jan-04-2002 7:10:00 Value can not be verified |
| Jan-01-2002 12:00:50 | 50 | Raw, Good | ANNOTATION: Jan-04-2002 7:00:00 Scanner fixed |
| Jan-01-2002 12:01:00 | 60 | Raw, Good |  |
| Jan-01-2002 12:01:10 | 70 | Raw, Uncertain | ANNOTATION: Jan-02-2002 8:00:00 Value flagged as questionable |
| Jan-01-2002 12:01:20 | 80 | Raw, Good |  |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
|  | NULL | No Data | No more entries, awaiting next scan. |

The example historian also has Annotations associated with three data points.

For the purposes of all Historian 1 examples:

- TreatUncertainAsBad = False.  Therefore Uncertain values are included in Aggregate call.

- Stepped attribute = False. Therefore SlopedInterpolation is used between data points.

- SteppedSlopedExtrapolation = False. Therefore SteppedExtrapolation is used at end boundary conditions

    - Example aggregate data – Historian 2

This example is included to illustrate non-periodic data.  For the purposes of Historian 2 examples consider a source historian with the following data:

| Timestamp | Value | StatusCode | Notes |
| --- | --- | --- | --- |
| Jan-01-2002 12:00:00 | - | Bad_NoData | First archive entry, Point Created |
| Jan-01-2002 12:00:02 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:25 | 20 | Raw, Good |  |
| Jan-01-2002 12:00:28 | 25 | Raw, Good |  |
| Jan-01-2002 12:00:39 | 30 | Raw, Good |  |
| Jan-01-2002 12:00:42 | 40 | Raw, Bad | Bad quality data received, Bad data entered |
| Jan-01-2002 12:00:48 | 40 | Raw, Good | Received good StatusCode value |
| Jan-01-2002 12:00:52 | 50 | Raw, Good |  |
| Jan-01-2002 12:01:12 | 60 | Raw, Good |  |
| Jan-01-2002 12:01:17 | 70 | Raw, Uncertain | Value is flagged as questionable |
| Jan-01-2002 12:01:23 | 70 | Raw, Good |  |
| Jan-01-2002 12:01:26 | 80 | Raw, Good |  |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
|  | - | No Data | No more entries, awaiting next Value. |

For the purposes of all Historian 2 examples:

- TreatUncertainAsBad = True.  Therefore Uncertain values are treated as Bad, and not included in the Aggregate call.

- Stepped attribute = False. Therefore SlopedInterpolation is used between data points.

- SteppedSlopedExtrapolation = False, Therefore SteppedExtrapolation is used at end boundary conditions

    - Example aggregate data – Historian 3

This example is included to illustrate stepped data.  For the purposes of Historian 3 examples consider a source historian with the following data:

| Timestamp | Value | StatusCode | Notes |
| --- | --- | --- | --- |
| Jan-01-2002 12:00:00 | - | Bad_NoData | First archive entry, Point Created |
| Jan-01-2002 12:00:02 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:25 | 20 | Raw, Good |  |
| Jan-01-2002 12:00:28 | 25 | Raw, Good |  |
| Jan-01-2002 12:00:39 | 30 | Raw, Good |  |
| Jan-01-2002 12:00:42 | 40 | Raw, Bad | Bad quality data received, Bad data entered |
| Jan-01-2002 12:00:48 | 40 | Raw, Good | Received good StatusCode value |
| Jan-01-2002 12:00:52 | 50 | Raw, Good |  |
| Jan-01-2002 12:01:12 | 60 | Raw, Good |  |
| Jan-01-2002 12:01:17 | 70 | Raw, Uncertain | Value is flagged as questionable |
| Jan-01-2002 12:01:23 | 70 | Raw, Good |  |
| Jan-01-2002 12:01:26 | 80 | Raw, Good |  |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
|  | - | No Data | No more entries, awaiting next Value. |

For the purposes of all Historian 3 examples:

- TreatUncertainAsBad = True.  Therefore Uncertain values are treated as Bad, and not included in the Aggregate call.

- Stepped attribute = True. Therefore SteppedInterpolation is used between data points.

- SteppedSlopedExtrapolation = False, Therefore SteppedExtrapolation is used at end boundary conditions

    - Example aggregate data – Historian 4

This example is included to illustrate Boolean data.  For the purposes of Historian 4 examples consider a source historian with the following data:

| Timestamp | Value | StatusCode | Notes |
| --- | --- | --- | --- |
| Jan-01-2002 12:00:00 | - | Bad_NoData | First archive entry, Point Created |
| Jan-01-2002 12:00:02 | TRUE | Raw, Good |  |
| Jan-01-2002 12:00:25 | FALSE | Raw, Good |  |
| Jan-01-2002 12:00:28 | TRUE | Raw, Good |  |
| Jan-01-2002 12:00:39 | TRUE | Raw, Good |  |
| Jan-01-2002 12:00:42 | FALSE | Raw, Bad | Bad quality data received, Bad data entered |
| Jan-01-2002 12:00:48 | TRUE | Raw, Good | Received good StatusCode value |
| Jan-01-2002 12:00:52 | FALSE | Raw, Good |  |
| Jan-01-2002 12:01:12 | FALSE | Raw, Good |  |
| Jan-01-2002 12:01:17 | TRUE | Raw, Uncertain | Value is flagged as questionable |
| Jan-01-2002 12:01:23 | TRUE | Raw, Good |  |
| Jan-01-2002 12:01:26 | FALSE | Raw, Good |  |
| Jan-01-2002 12:01:30 | TRUE | Raw, Good |  |
|  | - | No Data | No more entries, awaiting next Value. |

For the purposes of all Historian 4 examples:

- TreatUncertainAsBad = True.  Therefore Uncertain values are treated as Bad, and not included in the aggregate call.

- Stepped attribute = True. Therefore SteppedInterpolation is used between data points.

- SteppedSlopedExtrapolation = False, Therefore SteppedExtrapolation is used at end boundary conditions

  - Interpolative

    - Description

The following examples demonstrate Interpolative Aggregate scenarios.

    - Interpolated data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:10 | 10 | Raw, Good |  |
| Jan-01-02 12:00:15 | 15 | Interpolated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:10 | 13.5 | Interpolated, Good | Interpolated between values at 12:00:02 and 12:00:25 |
| Jan-01-02 12:00:15 | 15.7 | Interpolated, Good | Interpolated between values at 12:00:02 and 12:00:25 |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:10 | 10 | Interpolated, Good |  |
| Jan-01-02 12:00:15 | 10 | Interpolated, Good |  |

    - Interpolated data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:35 | 35 | Interpolated, Uncertain |  |
| Jan-01-02 12:00:40 | 40 | Interpolated, Uncertain | Raw Value is Bad |
| Jan-01-02 12:00:45 | 45 | Interpolated, Uncertain | Bounding Value Bad |
| Jan-01-02 12:00:50 | 50 | Raw, Good |  |
| Jan-01-02 12:00:55 | 55 | Interpolated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:35 | 28.2 | Interpolated, Good | Interpolated between values at 12:00:28 and 12:00:39 |
| Jan-01-02 12:00:40 | 31.1 | Interpolated, Uncertain | Interpolated between values at 12:00:39 and 12:00:48 |
| Jan-01-02 12:00:45 | 36.7 | Interpolated, Uncertain | Interpolated between values at 12:00:39 and 12:00:48 |
| Jan-01-02 12:00:50 | 45 | Interpolated, Good |  |
| Jan-01-02 12:00:55 | 51.5 | Interpolated, Good |  |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:35 | 25 | Interpolated, Good | Interpolated from value at 12:00:28 |
| Jan-01-02 12:00:40 | 30 | Interpolated, Good | Interpolated from value at 12:00:39 |
| Jan-01-02 12:00:45 | 30 | Interpolated, Uncertain | Interpolated from value at 12:00:39. Bad value at 12:00:42 ignored. |
| Jan-01-02 12:00:50 | 40 | Interpolated, Good |  |
| Jan-01-02 12:00:55 | 50 | Interpolated, Good |  |

    - Interpolated data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:01:20 | 80 | Raw, Good | Uncertain Values excluded. |
| Jan-01-02 12:01:25 | 85 | Interpolated, Good |  |
| Jan-01-02 12:01:30 | 90 | Raw, Good |  |
| Jan-01-02 12:01:35 | 90 | Interpolated, Uncertain | Bounding Value at 12:01:30, Extrapolated using stepped method |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:01:20 | 67.3* | Interpolated, Uncertain | Interpolated between values at 12:01:12 and 12:01:23 |
| Jan-01-02 12:01:25 | 76.7 | Interpolated, Good |  |
| Jan-01-02 12:01:30 | 90 | Raw, Good |  |
| Jan-01-02 12:01:35 | 90 | Interpolated, Uncertain | Bounding Value at 12:01:30, Extrapolated using stepped method |

* If Historian 2 had treated Uncertain values as Good.  The value would be 70, interpolated between 12:00:17 and 12:00:23 and the quality would be “Interpolated,Good”.

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:01:20 | 60* | Interpolated, Uncertain | Interpolated from value at 12:01:12 . Uncertain value at 12:01:17 ignored. |
| Jan-01-02 12:01:25 | 70 | Interpolated, Good |  |
| Jan-01-02 12:01:30 | 90 | Raw, Good |  |
| Jan-01-02 12:01:35 | 90 | Interpolated, Uncertain | Bounding Value at 12:01:30, Extrapolated using stepped method |

* If Historian 3 had treated Uncertain values as Good.  The value would be 70, interpolated from 12:00:17 and the quality would be “Interpolated,Good”.

    - Interpolated data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad | No bounding Value, do not extrapolate |
| Jan-01-2002 12:00:05 | - | No Data, Bad | No bounding value, do not extrapolate |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | 15 | Interpolated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad | No bounding Value, do not extrapolate |
| Jan-01-2002 12:00:05 | 11.3 | Interpolated, Good | Interpolated between values at 12:00:02 and 12:00:25 |
| Jan-01-2002 12:00:10 | 13.5 | Interpolated, Good | Interpolated between values at 12:00:02 and 12:00:25 |
| Jan-01-2002 12:00:15 | 15.7 | Interpolated, Good | Interpolated between values at 12:00:02 and 12:00:25 |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad | No bounding Value, do not extrapolate |
| Jan-01-2002 12:00:05 | 10 | Interpolated, Good | Interpolated from value at 12:00:02 |
| Jan-01-2002 12:00:10 | 10 | Interpolated, Good | Interpolated from value at 12:00:02 |
| Jan-01-2002 12:00:15 | 10 | Interpolated, Good | Interpolated from value at 12:00:02 |

  - Average

    - Description

The following examples demonstrate Average Aggregate scenarios.

    - Average data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 10 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad | No Raw data in intervals |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | - | No Data, Bad | No Raw data in interval |
| Jan-01-2002 12:00:15 | - | No Data, Bad | No Raw data in intervals |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | - | No Data, Bad | No Raw data in interval |
| Jan-01-2002 12:00:15 | - | No Data, Bad | No Raw data in interval |

    - Average data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | - | No Data, Bad |  |
| Jan-01-2002 12:00:40 | - | No Data, Bad | Only Bad data in interval |
| Jan-01-2002 12:00:45 | - | No Data, Bad | No data in interval |
| Jan-01-2002 12:00:50 | 50 | Calculated, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad | No data in intervals |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 30 | Calculated, Good |  |
| Jan-01-2002 12:00:40 | - | No Data, Bad | No data in interval |
| Jan-01-2002 12:00:45 | 40 | Calculated, Good |  |
| Jan-01-2002 12:00:50 | 50 | Calculated, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad | No data in intervals |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 30 | Calculated, Good |  |
| Jan-01-2002 12:00:40 | - | No Data, Bad | No data in interval |
| Jan-01-2002 12:00:45 | 40 | Calculated, Good |  |
| Jan-01-2002 12:00:50 | 50 | Calculated, Good | No data in intervals |
| Jan-01-2002 12:00:55 | - | No Data, Bad |  |

    - Average data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 80 | Calculated, Good |  |
| Jan-01-2002 12:01:25 | - | No Data, Bad | No data in interval |
| Jan-01-2002 12:01:30 | 90 | Calculated, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad | No data in intervals |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 70 | Calculated, Good |  |
| Jan-01-2002 12:01:25 | 80 | Calculated, Good |  |
| Jan-01-2002 12:01:30 | 90 | Calculated, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad | No data in intervals |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 70 | Calculated, Good |  |
| Jan-01-2002 12:01:25 | 80 | Calculated, Good |  |
| Jan-01-2002 12:01:30 | 90 | Calculated, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad | No data in intervals |

    - Average data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad | No data in interval |
| Jan-01-2002 12:00:05 | - | No Data, Bad | No data in intervals |
| Jan-01-2002 12:00:10 | 10 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad | No data in intervals |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | 10 | Partial, Good | Partial interval :02-:05 |
| Jan-01-2002 12:00:05 | - | No Data, Bad | No data in intervals |
| Jan-01-2002 12:00:10 | - | No Data, Bad | No data in interval |
| Jan-01-2002 12:00:15 | - | No Data, Bad | No data in intervals |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | 10 | Partial, Good | Partial interval :02-:05 |
| Jan-01-2002 12:00:05 | - | No Data, Bad | No data in intervals |
| Jan-01-2002 12:00:10 | - | No Data, Bad | No data in interval |
| Jan-01-2002 12:00:15 | - | No Data, Bad | No data in intervals |

  - TimeAverage

    - Description

The following examples demonstrate TimeAverage Aggregate scenarios.

All cases use the interpolated values determined in Cases outlined in section A.2 for the bounding values.

    - TimeAverage data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 12.5 | Calculated, Good | Area under the line between 12:00:10 and 12:00:15 divided by interval length of 5 |
| Jan-01-2002 12:00:15 | 17.5 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 14.5 | Calculated, Good | Area under the line between 12:00:10 and 12:00:15 divided by interval length of 5 |
| Jan-01-2002 12:00:15 | 16.7 | Calculated, Good |  |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 10 | Calculated, Good | Area under the line between 12:00:10 and 12:00:15 divided by interval length of 5 |
| Jan-01-2002 12:00:15 | 10 | Calculated, Good |  |

    - TimeAverage data with good bounding value  with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 37.5 | Calculated, Uncertain | Interpolate values at :35 and :40 using bounds at :30 and :50 Uncertain means Bad Value ignored |
| Jan-01-2002 12:00:40 | 42.5 | Calculated, Uncertain | Interpolate values at :40 and :45 using bounds at :30 and :50 Uncertain means Bad Value ignored |
| Jan-01-2002 12:00:45 | 47.5 | Calculated, Uncertain | Interpolate value at :45 using bounds at :30 and :50 Used Raw value :50 Uncertain means Bad Value ignored |
| Jan-01-2002 12:00:50 | 52.5 | Calculated, Good | used Raw value at :50 and Interpolate :55 using bounds at :50 and 01:00 |
| Jan-01-2002 12:00:55 | 57.5 | Calculated, Good | Interpolate value at :55 using bounds at :50 and 01:00, Used raw value 01:00 |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 29.4 | Calculated, Uncertain | Interpolate value at :35 using bounds at :28 and :39 used raw Value at :39 Interpolate value at :40 using bounds at :29 and :48 Uncertain means Bad Value ignored |
| Jan-01-2002 12:00:40 | 33.9 | Calculated Uncertain | Interpolate values at :40 and :45 using bounds at :39 and :48 Uncertain means Bad Value ignored |
| Jan-01-2002 12:00:45 | 40.0 | Calculated Uncertain | Interpolate value at :45 using bounds at :39 and :48 Used Raw value at :48 Interpolate Value at :50 using bounds at :48 and :52 Uncertain means Bad Value ignored |
| Jan-01-2002 12:00:50 | 49.5 | Calculated, Good | Interpolate value at :50 using bounds at :48 and :52 Used Raw Value at 52 Interpolate Value at :55 using bounds at :52 and :01:12 |
| Jan-01-2002 12:00:55 | 52.8 | Calculated, Good | Interpolate Value at :55 and at 01:00 using bounds at :52 and :01:12 |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 26 | Calculated, Good | Interpolate value of 25 at :35 for interval of 4. Interpolate value of 30 at :39 for interval of 1. |
| Jan-01-2002 12:00:40 | 30 | Calculated Uncertain | Interpolate value of 30 at :35 for interval of 5. Uncertain means Bad Value ignored. |
| Jan-01-2002 12:00:45 | 34 | Calculated Uncertain | Interpolate value of 30 at :35 for interval of 3. Interpolate value of 40 at :48 for interval of 2. Uncertain means Bad Value ignored. |
| Jan-01-2002 12:00:50 | 46 | Calculated, Good | Interpolate value of 40 at :48 for interval of 2. Interpolate value of 50 at :52 for interval of 3. |
| Jan-01-2002 12:00:55 | 50 | Calculated, Good |  |

    - TimeAverage data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 82.5 | Calculated, Good | Value1– Interpolate value at :25 using bounds at :20 and :30 Value2– Interpolate value at :20 using bounds at :16 and :23 (Uncertain value at :17 is ignored by this historian) Interpolate Value at :25 using bounds at :23 and :26 |
| Jan-01-2002 12:01:25 | 87.5 | Calculated, Good | Value1– Interpolate value at :25 using bounds at :20 and :30 Value2– Interpolate value at :25 using bounds at :23 and :26 |
| Jan-01-2002 12:01:30 | 90* | Calculated, Uncertain | Extrapolate Value at :35 using value at :30 |
| Jan-01-2002 12:01:35 | 90* | Calculated, Uncertain | Extrapolate Values at :35 and :40 using value at :30 |

* SteppedExtrapolation is used at the boundary.  Servers may opt to Extrapolate data using SlopedExtrapolation.

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 70.5 | Calculated Uncertain | Interpolate value at :20 using bounds at :12 and :23 (Uncertain value at :17 is ignored by this historian) Used raw value at :23 Interpolate Value at :25 using bounds at :23 and :26 |
| Jan-01-2002 12:01:25 | 82.3 | Calculated, Good | Value2– Interpolate value at :25 using bounds at :23 and :26 Used raw value at :26 Used raw value at :30 |
| Jan-01-2002 12:01:30 | 90* | Calculated, Uncertain | Extrapolate Value at :35 using value at :30 |
| Jan-01-2002 12:01:35 | 90* | Calculated, Uncertain | Extrapolate Values at :35 and :40 using value at :30 |

* SteppedExtrapolation is used at the boundary.  Servers may opt to Extrapolate data using SlopedExtrapolation which would give values of 96.25 and 108.75 respectively.

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 64 | Calculated Uncertain | Interpolate value of 60 from :12 for interval of 3 Interpolate value of 70 from :17 for interval of 2 (Uncertain value at :17 is ignored by this historian) |
| Jan-01-2002 12:01:25 | 78 | Calculated, Good | Interpolate value of 70 from :23 for interval of 1 Interpolate value of 80 from :26 for interval of 4 |
| Jan-01-2002 12:01:30 | 90* | Calculated, Uncertain | Extrapolate Value at :35 from value at :30 |
| Jan-01-2002 12:01:35 | 90* | Calculated, Uncertain | Extrapolate Values at :35 and :40 from value at :30 |

* SteppedExtrapolation is used at the boundary.

    - TimeAverage data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Timestamp | Historian 1 |  | Historian 2 |  | Notes |
| --- | --- | --- | --- | --- | --- |
|  | Value | StatusCode | Value | StatusCode |  |
| Jan-01-2002 12:00:00 | - | No Data, Bad | 10.7 | Partial, Uncertain | Value1-No bounding value, do not extrapolate. No data in the interval Value2- Interpolate value at :05 using bounds at :02 and :25 Use partial interval :02 to :05, with interval of 3. |
| Jan-01-2002 12:00:05 | - | No Data, Bad | 12.4 | Calculated, Good | Value1-No bounding value, do not extrapolate. No data in the interval Value2- Interpolate values at :05 and 10 using bounds at :02 and :25 |
| Jan-01-2002 12:00:10 | 12.5 | Calculated, Good | 14.5 | Calculated, Good | Value1– Interpolate value at :15 using bounds at :10 and :20 Value2– Interpolate values at :10 and :15 using bounds at :02 and :25 |
| Jan-01-2002 12:00:15 | 17.5 | Calculated, Good | 16.7 | Calculated, Good | Value1– Interpolate value at :15 using bounds at :10 and :20 Value2– Interpolate values at :15 and :20 using bounds at :02 and :25 |

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad | No bounding value, do not extrapolate. No data in the interval |
| Jan-01-2002 12:00:05 | - | No Data, Bad | No bounding value, do not extrapolate. No data in the interval |
| Jan-01-2002 12:00:10 | 12.5 | Calculated, Good | Interpolate value at :15 using bounds at :10 and :20 |
| Jan-01-2002 12:00:15 | 17.5 | Calculated, Good | Interpolate value at :15 using bounds at :10 and :20 |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | 10.7 | Partial, Uncertain | Interpolate value at :05 using bounds at :02 and :25 Use partial interval :02 to :05, with interval of 3. |
| Jan-01-2002 12:00:05 | 12.4 | Calculated, Good | Interpolate values at :05 and 10 using bounds at :02 and :25 |
| Jan-01-2002 12:00:10 | 14.5 | Calculated, Good | Interpolate values at :10 and :15 using bounds at :02 and :25 |
| Jan-01-2002 12:00:15 | 16.7 | Calculated, Good | Interpolate values at :15 and :20 using bounds at :02 and :25 |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | 10 | Partial, Uncertain | Interpolate value of 10 from :02 Use partial interval :02 to :05, with interval of 3. |
| Jan-01-2002 12:00:05 | 10 | Calculated, Good |  |
| Jan-01-2002 12:00:10 | 10 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | 10 | Calculated, Good |  |

  - Total

    - Description

The following examples demonstrate Total Aggregate scenarios.

    - Total data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:10 | 10 | Raw, Good |  |
| Jan-01-02 12:00:15 | 0 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:10 | 10 | Calculated, Good |  |
| Jan-01-02 12:00:15 | 0 | Calculated, Good |  |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:10 | 10 | Calculated, Good |  |
| Jan-01-02 12:00:15 | 0 | Calculated, Good |  |

    - Total data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:35 | 0 | Calculated, Good |  |
| Jan-01-02 12:00:40 | - | No Data, Bad |  |
| Jan-01-02 12:00:45 | - | No Data, Bad |  |
| Jan-01-02 12:00:50 | 50 | Calculated, Good |  |
| Jan-01-02 12:00:55 | 0 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:35 | 30 | Calculated, Good |  |
| Jan-01-02 12:00:40 | - | No Data, Bad |  |
| Jan-01-02 12:00:45 | 40 | Calculated, Good |  |
| Jan-01-02 12:00:50 | 50 | Calculated, Good |  |
| Jan-01-02 12:00:55 | 0 | Calculated, Good |  |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:35 | 30 | Calculated, Good |  |
| Jan-01-02 12:00:40 | - | No Data, Bad |  |
| Jan-01-02 12:00:45 | 40 | Calculated, Good |  |
| Jan-01-02 12:00:50 | 50 | Calculated, Good |  |
| Jan-01-02 12:00:55 | 0 | Calculated, Good |  |

    - Total data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:01:20 | 80 | Calculated, Good |  |
| Jan-01-02 12:01:25 | 0 | Calculated, Good |  |
| Jan-01-02 12:01:30 | 90 | Calculated, Good |  |
| Jan-01-02 12:01:35 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:01:20 | 70 | Calculated, Good |  |
| Jan-01-02 12:01:25 | 80 | Calculated, Good |  |
| Jan-01-02 12:01:30 | 90 | Calculated, Good |  |
| Jan-01-02 12:01:35 | - | No Data, Bad |  |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:01:20 | 70 | Calculated, Good |  |
| Jan-01-02 12:01:25 | 80 | Calculated, Good |  |
| Jan-01-02 12:01:30 | 90 | Calculated, Good |  |
| Jan-01-02 12:01:35 | - | No Data, Bad |  |

  - TotalizeAverage

    - Description

The following examples demonstrate TotalizeAverage Aggregate scenarios.

    - TotalizeAverage data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 62.5 | Calculated, Good | Area under the line between 12:00:10 and 12:00:15 |
| Jan-01-2002 12:00:15 | 87.5 | Calculated, Good |  |

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 72.5 | Calculated, Good | Area under the line between 12:00:10 and 12:00:15 |
| Jan-01-2002 12:00:15 | 83.5 | Calculated, Good |  |

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 50 | Calculated, Good | Area under the line between 12:00:10 and 12:00:15 |
| Jan-01-2002 12:00:15 | 50 | Calculated, Good |  |

  - Minimum

    - Description

The following examples demonstrate Minimum Aggregate scenarios.

    - Minimum data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | - | No Data, Bad |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

    - Minimum data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | - | No Data, Bad |  |
| Jan-01-2002 12:00:40 | - | No Data, Bad | Only Bad data in interval. |
| Jan-01-2002 12:00:45 | - | No Data, Bad |  |
| Jan-01-2002 12:00:50 | 50 | Raw, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 30 | Calculated, Good |  |
| Jan-01-2002 12:00:40 | 40 | Calculated, Bad |  |
| Jan-01-2002 12:00:45 | 40 | Calculated, Good |  |
| Jan-01-2002 12:00:50 | 50 | Calculated, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad |  |

    - Minimum data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 80 | Raw, Good |  |
| Jan-01-2002 12:01:25 | - | No Data, Bad |  |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 70 | Calculated Good |  |
| Jan-01-2002 12:01:25 | 80 | Calculated Good |  |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad |  |

    - Minimum data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad |  |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | 10 | Calculated Good |  |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:10 | - | No Data, Bad |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

    - Minimum data with Partial Interval.

Start:  Jan-01-2002 12:00:05  End:  Jan-01-2002 12:00:35  Interval: 00:00:16

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:05 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:21 | 30 | Partial, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:21 | 20 | Partial, Good |  |

  - Maximum

    - Description

The following examples demonstrate Maximum Aggregate scenarios.

    - Maximum data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | - | No Data, Bad |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

    - Maximum data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | - | No Data, Bad |  |
| Jan-01-2002 12:00:40 | - | No Data, Bad | Only Bad data in interval. |
| Jan-01-2002 12:00:45 | - | No Data, Bad |  |
| Jan-01-2002 12:00:50 | 50 | Raw, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 30 | Calculated, Good |  |
| Jan-01-2002 12:00:40 | 40 | Calculated, Bad | Only Bad data in interval. |
| Jan-01-2002 12:00:45 | 40 | Calculated, Good |  |
| Jan-01-2002 12:00:50 | 50 | Calculated, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad |  |

    - Maximum data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 80 | Raw, Good |  |
| Jan-01-2002 12:01:25 | - | No Data, Bad |  |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 70 | Calculated Good |  |
| Jan-01-2002 12:01:25 | 80 | Calculated Good |  |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad |  |

    - Maximum data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad |  |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | 10 | Calculated Good |  |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:10 | - | No Data, Bad |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

    - Maximum data with Partial Interval.

Start:  Jan-01-2002 12:00:05  End:  Jan-01-2002 12:00:35  Interval: 00:00:16

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:05 | 20 | Raw, Good |  |
| Jan-01-2002 12:00:21 | 30 | Partial, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:21 | 25 | Partial, Good |  |

  - MininumActualTime

    - Description

The following examples demonstrate the MinimumActualTime Aggregate scenarios.

    - MininumActualTime data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad | No raw data in interval, do not interpolate |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | - | No Data, Bad | No raw data in interval, do not interpolate |
| Jan-01-2002 12:00:15 | - | No Data, Bad | No raw data in interval, do not interpolate |

    - MininumActualTime data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | - | No Data, Bad | No raw data in interval, do not interpolate |
| Jan-01-2002 12:00:40 | - | No Data, Bad | No raw data in interval, do not interpolate |
| Jan-01-2002 12:00:45 | - | No Data, Bad | No raw data in interval, do not interpolate |
| Jan-01-2002 12:00:50 | 50 | Raw, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad | No raw data in interval, do not interpolate |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:39 | 30 | Raw, Good |  |
| Jan-01-2002 12:00:42 | - | Raw, Bad | Only Bad data in interval |
| Jan-01-2002 12:00:48 | 40 | Raw, Good |  |
| Jan-01-2002 12:00:52 | 50 | Raw, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad | No raw data in interval, do not interpolate |

    - MininumActualTime data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 80 | Raw, Good |  |
| Jan-01-2002 12:01:25 | - | No Data, Bad | No raw data in interval, do not interpolate |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:23 | 70 | Raw, Good |  |
| Jan-01-2002 12:01:26 | 80 | Raw, Good |  |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad |  |

    - MininumActualTime data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad |  |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:02 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:10 | - | No Data, Bad |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

    - MininumActualTime  with Partial Interval.

Start:  Jan-01-2002 12:00:05  End:  Jan-01-2002 12:00:35  Interval: 00:00:16

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:30 | 30 | Partial, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:25 | 20 | Partial, Good |  |

  - MaximumActualTime

    - Description

The following examples demonstrate MaximumActualTime Aggregate scenarios.

    - MaximumActualTime data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad | No raw data in interval, do not interpolate |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | - | No Data, Bad | No raw data in interval, do not interpolate |
| Jan-01-2002 12:00:15 | - | No Data, Bad | No raw data in interval, do not interpolate |

    - MaximumActualTime data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | - | No Data, Bad | No raw data in interval, do not interpolate |
| Jan-01-2002 12:00:40 | - | No Data, Bad | No raw data in interval, do not interpolate |
| Jan-01-2002 12:00:45 | - | No Data, Bad | No raw data in interval, do not interpolate |
| Jan-01-2002 12:00:50 | 50 | Raw, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad | No raw data in interval, do not interpolate |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:39 | 30 | Raw, Good |  |
| Jan-01-2002 12:00:42 | - | No Data, Bad | Only Bad data in interval |
| Jan-01-2002 12:00:48 | 40 | Raw, Good |  |
| Jan-01-2002 12:00:52 | 50 | Raw, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad | No raw data in interval, do not interpolate |

    - MaximumActualTime data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 80 | Raw, Good |  |
| Jan-01-2002 12:01:25 | - | No Data, Bad | No raw data in interval, do not interpolate |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:23 | 70 | Raw, Good |  |
| Jan-01-2002 12:01:26 | 80 | Raw, Good |  |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
| Jan-01-2002 12:01:35 | - | No Data, Bad |  |

    - MaximumActualTime data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad |  |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:02 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:10 | - | No Data, Bad |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad |  |

    - MaximumActualTime  with Partial Interval.

Start:  Jan-01-2002 12:00:05  End:  Jan-01-2002 12:00:35  Interval: 00:00:16

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:20 | 20 | Raw, Good |  |
| Jan-01-2002 12:00:30 | 30 | Partial, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:05 | - | No Data, Bad |  |
| Jan-01-2002 12:00:28 | 25 | Partial, Good |  |

  - AnnotationCount

    - Description

The following examples demonstrate AnnotationCount Aggregate scenarios.

    - Annotations on various data quality

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:02:00  Interval: 00:01:00

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:00 | 3 | Calculated, Good |  |
| Jan-01-02 12:01:00 | 1 | Calculated, Good |  |

  - Count

    - Description

The following examples demonstrate Count Aggregate scenarios.

    - Count data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 1 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | 0 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 0 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | 0 | Calculated, Good |  |

    - Count data with uncertain data in the interval.

Start:  Jan-01-2002 12:00:50  End:  Jan-01-2002 12:01:30  Interval: 00:00:00

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:50 | 4* | Calculated, Good |  |

* For servers with  TreatUncertainAsBad = True then the result would be 3.

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:50 | 4 | Calculated, Uncertain | Treats uncertain as bad, which also changes StatusCode |

  - DurationInState0

    - Description

The following examples demonstrate DurationInState0 Aggregate scenarios.

    - Interpolated data with good bounding value.

Start:  Jan-01-2002 12:00:20  End:  Jan-01-2002 12:00:30  Interval: 00:00:05

| Historian 4 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:20 | 0 | Interpolated, Good | Value interpolated from 12:00:02. Value in 1 state for full interval |
| Jan-01-02 12:00:25 | 3000 | Raw, Good | Raw value at 12:00:25. Value transitions to 1 state at 12:00:28 |

Duration in milliseconds

    - Interpolated data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 4 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:35 | 0 | Interpolated, Good | Interpolated from value at 12:00:28. |
| Jan-01-02 12:00:40 | 0 | Interpolated, Uncertain | Interpolated from value at 12:00:39. Bad value ignored 12:00:40- :45 |
| Jan-01-02 12:00:45 | 0 | Interpolated, Uncertain | Interpolated from value at 12:00:39. Bad value ignored 12:00:45- :48 |
| Jan-01-02 12:00:50 | 3000 | Interpolated, Good | Interpolate to :50, raw value at :52 |
| Jan-01-02 12:00:55 | 5000 | Interpolated, Good |  |

Duration in milliseconds

    - Interpolated data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 4 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:01:20 | 3000* | Interpolated, Uncertain | Interpolated from value at 12:01:12 . Uncertain value at 12:01:17 ignored. |
| Jan-01-02 12:01:25 | 4000 | Interpolated, Good |  |
| Jan-01-02 12:01:30 | 0 | Raw, Good |  |
| Jan-01-02 12:01:35 | 0 | Interpolated, Uncertain | Bounding Value at 12:01:30, Extrapolated using stepped method |

* If Historian 4 had treated Uncertain values as Good.  The value would be 0, interpolated from 12:00:17 and the quality would be “Interpolated,Good”.

    - Interpolated data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 4 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad | No bounding Value, do not extrapolate |
| Jan-01-2002 12:00:05 | 0 | Interpolated, Good | Interpolated from value at 12:00:02 |
| Jan-01-2002 12:00:10 | 0 | Interpolated, Good | Interpolated from value at 12:00:02 |
| Jan-01-2002 12:00:15 | 0 | Interpolated, Good | Interpolated from value at 12:00:02 |

  - DurationInState1

    - Description

The following examples demonstrate DurationInState1 Aggregate scenarios.

    - Interpolated data with good bounding value.

Start:  Jan-01-2002 12:00:20  End:  Jan-01-2002 12:00:30  Interval: 00:00:05

| Historian 4 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:20 | 5000 | Interpolated, Good | Value interpolated from 12:00:02. |
| Jan-01-02 12:00:25 | 2000 | Raw, Good | Raw value at 12:00:25. Value transitions to 1 state at 12:00:28 |

Duration in milliseconds

    - Interpolated data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 4 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:35 | 5000 | Interpolated, Good | Interpolated from value at 12:00:28. |
| Jan-01-02 12:00:40 | 5000 | Interpolated, Uncertain | Interpolated from value at 12:00:39. Bad value ignored 12:00:42- :45 |
| Jan-01-02 12:00:45 | 5000 | Interpolated, Uncertain | Interpolated from value at 12:00:39. Bad value ignored 12:00:45- :48 |
| Jan-01-02 12:00:50 | 2000 | Interpolated, Good |  |
| Jan-01-02 12:00:55 | 0 | Interpolated, Good |  |

Duration in milliseconds

    - Interpolated data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 4 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:01:20 | 2000* | Interpolated, Uncertain | Interpolated from value at 12:01:12 . Uncertain value at 12:01:17 ignored. |
| Jan-01-02 12:01:25 | 1000 | Interpolated, Good |  |
| Jan-01-02 12:01:30 | 5000 | Raw, Good |  |
| Jan-01-02 12:01:35 | 5000 | Interpolated, Uncertain | Bounding Value at 12:01:30, Extrapolated using stepped method |

* If Historian 4 had treated Uncertain values as Good.  The value would be 5000, interpolated from 12:00:17 and the quality would be “Interpolated,Good”.

    - Interpolated data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 4 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | 3000 | Interpolated, Uncertain | 12:00:00-12:00:02 ignored. No bounding Value, do not extrapolate |
| Jan-01-2002 12:00:05 | 5000 | Interpolated, Good | Interpolated from value at 12:00:02 |
| Jan-01-2002 12:00:10 | 5000 | Interpolated, Good | Interpolated from value at 12:00:02 |
| Jan-01-2002 12:00:15 | 5000 | Interpolated, Good | Interpolated from value at 12:00:02 |

  - NumberOfTransitions

    - Description

The following examples demonstrate NumberOfTransitions Aggregate scenarios.

    - Interpolated data with good bounding value.

Start:  Jan-01-2002 12:00:20  End:  Jan-01-2002 12:00:30  Interval: 00:00:05

| Historian 4 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:20 | 0 | Interpolated, Good | Value interpolated from 12:00:02. |
| Jan-01-02 12:00:25 | 1 | Raw, Good | Raw value at 12:00:25. Value transitions to 1 state at 12:00:28 |

    - Interpolated data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:35 | 0 | Interpolated, Good | Interpolated from value at 12:00:28. |
| Jan-01-02 12:00:40 | 0 | Interpolated, Uncertain | Interpolated from value at 12:00:39. Bad value ignored 12:00:42- :45 |
| Jan-01-02 12:00:45 | 0 | Interpolated, Uncertain | Interpolated from value at 12:00:39. Bad value ignored 12:00:45- :48 |
| Jan-01-02 12:00:50 | 1 | Interpolated, Good |  |
| Jan-01-02 12:00:55 | 0 | Interpolated, Good |  |

    - Interpolated data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 3 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:01:20 | 1* | Interpolated, Uncertain | Interpolated from value at 12:01:12 . Uncertain value at 12:01:17 ignored. |
| Jan-01-02 12:01:25 | 1 | Interpolated, Good | Transistion at 12:01:30 not included in this interval |
| Jan-01-02 12:01:30 | 1 | Raw, Good | Transistion at 12:01:30 included in this interval |
| Jan-01-02 12:01:35 | 0 | Interpolated, Uncertain | Bounding Value at 12:01:30, Extrapolated using stepped method |

* If Historian 4 had treated Uncertain values as Good.  The value would be 0, interpolated from 12:00:17 and the quality would be “Interpolated,Good”.

    - Interpolated data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | 0 | Interpolated, Uncertain | 12:00:00-12:00:02 ignored. No bounding Value, do not extrapolate |
| Jan-01-2002 12:00:05 | 0 | Interpolated, Good | Interpolated from value at 12:00:02 |
| Jan-01-2002 12:00:10 | 0 | Interpolated, Good | Interpolated from value at 12:00:02 |
| Jan-01-2002 12:00:15 | 0 | Interpolated, Good | Interpolated from value at 12:00:02 |

  - Start

    - Description

The following examples demonstrate Start Aggregate scenarios.

    - Start data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | - | No Data, Bad | Return Timestamp of the interval |

    - Start data with bad bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | - | No Data, Bad | Return Timestamp of the interval |
| Jan-01-2002 12:00:15 | - | No Data, Bad | Return Timestamp of the interval |

    - Start data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | - | No Data, Bad |  |
| Jan-01-2002 12:00:40 | 40 | Raw, Bad | Raw Value (If Bad values are stored) |
| Jan-01-2002 12:00:45 | - | No Data, Bad |  |
| Jan-01-2002 12:00:50 | 50 | Raw, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:39 | 30 | Raw, Good | First raw in :35-:40 at :39 |
| Jan-01-2002 12:00:42 | 40 | Raw, Bad | Raw Value (If Bad values are stored) |
| Jan-01-2002 12:00:48 | 40 | Raw, Good | First raw in :45-:50 at :48 |
| Jan-01-2002 12:00:52 | 50 | Raw, Good | First raw in :50-:55 at :52 |
| Jan-01-2002 12:00:55 | - | No Data, Bad |  |

    - Start  data with partial intervals.

Start:  Jan-01-2002 12:00:05  End:  Jan-01-2002 12:00:35  Interval: 00:00:16

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 10 | Raw, Good | First raw in :05-:21 at :10 |
| Jan-01-2002 12:00:30 | 30 | Partial, Good | First raw in :21-:35 at :30 |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:25 | - | No Data, Bad | No raw data in :05-:21 at :10 |
| Jan-01-2002 12:00:25 | 20 | Raw, Good | First raw in :21-:35 at :25 |

  - End

    - Description

The following examples demonstrate End Aggregate scenarios.

    - End data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 10 | Raw, Good | Last raw in :10-:15 at :10 |
| Jan-01-2002 12:00:15 | - | No Data, Bad | Return Timestamp of the interval. |

    - End data with bad bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | - | No Data, Bad | Return Timestamp of the interval |
| Jan-01-2002 12:00:15 | - | No Data, Bad | Return Timestamp of the interval |

    - End data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | - | No Data, Bad |  |
| Jan-01-2002 12:00:40 | 40 | Raw, Bad | Raw Value (If Bad values are stored) |
| Jan-01-2002 12:00:45 | - | No Data, Bad |  |
| Jan-01-2002 12:00:50 | 50 | Raw, Good |  |
| Jan-01-2002 12:00:55 | - | No Data, Bad |  |

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:39 | 30 | Raw, Good | Last raw in :35-:40 at :39 |
| Jan-01-2002 12:00:40 | 40 | Raw, Bad | Raw Value (If Bad values are stored) |
| Jan-01-2002 12:00:48 | 40 | Raw, Good | Last raw in :45-:50 at :48 |
| Jan-01-2002 12:00:52 | 50 | Raw, Good | Last raw in :50-:55 at :52 |
| Jan-01-2002 12:00:55 | - | No Data, Bad |  |

    - End  data with partial intervals.

Start:  Jan-01-2002 12:00:05  End:  Jan-01-2002 12:00:35  Interval: 00:00:16

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 20 | Raw, Good | Last raw in :05-:21 at :10 |
| Jan-01-2002 12:00:30 | 30 | Partial, Good | Last raw in :21-:35 at :30 |

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:25 | - | No Data, Bad | No raw data in :05-:21 at :10 |
| Jan-01-2002 12:00:28 | 25 | Raw, Good | Last raw in :21-:35 at :28 |

  - DurationGood

    - Description

The following examples demonstrate DurationGood Aggregate scenarios.  Duration values are in milliseconds.

    - DurationGood data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | 5000 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | 5000 | Calculated, Good |  |

    - DurationGood data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:00:40 | 0 | Calculated, Good |  |
| Jan-01-2002 12:00:45 | 0 | Calculated, Good |  |
| Jan-01-2002 12:00:50 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:00:55 | 5000 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 5000 | Calculated, Good | Value2-Good from :35 to :39. Good :39 to :40 |
| Jan-01-2002 12:00:40 | 2000 | Calculated, Good | Value2-Good from :40 to :42. Bad :42 to :45 |
| Jan-01-2002 12:00:45 | 2000 | Calculated, Good | Value2-Bad from :45 to :48. Good :48 to :50 |
| Jan-01-2002 12:00:50 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:00:55 | 5000 | Calculated, Good |  |

    - DurationGood data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:01:25 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:01:30 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:01:35 | 5000 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:20 | 2000 | Calculated, Good | Value2-Uncertain from :20 to :23. Good :23 to :25 |
| Jan-01-2002 12:01:25 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:01:30 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:01:35 | 5000 | Calculated, Good |  |

    - DurationGood data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | 0 | Calculated, Good | No bound, Bad from :00 to :05 |
| Jan-01-2002 12:00:05 | 0 | Calculated, Good | No bound, Bad from :05 to :10 |
| Jan-01-2002 12:00:10 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | 5000 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | 3000 | Calculated, Good | Bad from :00 to :02. Good :02 to :05 |
| Jan-01-2002 12:00:05 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:00:10 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | 5000 | Calculated, Good |  |

    - DurationGood data with uncertain data in the interval.

Start:  Jan-01-2002 12:01:00  End:  Jan-01-2002 12:01:30  Interval: 00:00:00

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:00 | 20000* | Calculated, Good | Uncertain from :10 to :20 |

* Uncertain data should not be counted as good.

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:00 | 25000* | Calculated, Good | Uncertain from :12 to :17 |

* Uncertain data should not be counted as good.

  - DurationBad

    - Description

The following examples demonstrate DurationBad Aggregate scenarios.

    - DurationBad data with good bounding value.

Start:  Jan-01-2002 12:00:10  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 0 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | 0 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:10 | 0 | Calculated, Good |  |
| Jan-01-2002 12:00:15 | 0 | Calculated, Good |  |

    - DurationBad data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 0 | Calculated, Good |  |
| Jan-01-2002 12:00:40 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:00:45 | 5000 | Calculated, Good |  |
| Jan-01-2002 12:00:50 | 0 | Calculated, Good |  |
| Jan-01-2002 12:00:55 | 0 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:35 | 0 | Calculated, Good |  |
| Jan-01-2002 12:00:40 | 3000 | Calculated, Good |  |
| Jan-01-2002 12:00:45 | 3000 | Calculated, Good |  |
| Jan-01-2002 12:00:50 | 0 | Calculated, Good |  |
| Jan-01-2002 12:00:55 | 0 | Calculated, Good |  |

    - DurationBad data with uncertain data in the interval.

Start:  Jan-01-2002 12:01:00  End:  Jan-01-2002 12:01:30  Interval: 00:00:00

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:00 | 0 | Calculated, Good |  |

| Historian 2 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:01:00 | 0 | Calculated, Good |  |

- Aggregate Specific Examples – Monitoried Item

  - Real-time Aggregate specific characteristics

    - Example aggregate data – Real-time Source

For the purposes of Real-time Source examples consider an input stream with the following data:

| Timestamp | Value | StatusCode | Notes |
| --- | --- | --- | --- |
| Jan-01-2002 12:00:00 | - | Bad_NoData | Monitored Item not configured yet. |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:20 | 20 | Raw, Good |  |
| Jan-01-2002 12:00:30 | 30 | Raw, Good |  |
| Jan-01-2002 12:00:40 | 40 | Raw, Bad |  |
| Jan-01-2002 12:00:50 | 50 | Raw, Good |  |
| Jan-01-2002 12:01:00 | 60 | Raw, Good |  |
| Jan-01-2002 12:01:10 | 70 | Raw, Uncertain |  |
| Jan-01-2002 12:01:20 | 80 | Raw, Good |  |
| Jan-01-2002 12:01:30 | 90 | Raw, Good |  |
|  | NULL | No Data | No more entries, awaiting next scan. |

  - Interpolative

    - Description

The following examples demonstrate Interpolative Aggregate scenarios.

    - Interpolated data with good bounding value.

Start:  Jan-01-2002 12:00:10  Sampling Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:10 | 10 | Raw, Good |  |
| Jan-01-02 12:00:15 | 15 | Interpolated, Good |  |
| Jan-01-02 12:00:20 | 20 | Raw, Good |  |
| Jan-01-02 12:00:25 | 25 | Interpolated, Good |  |
| Jan-01-02 12:00:30 | 30 | Raw, Good |  |
| Jan-01-02 12:00:35 | 35 | Interpolated, Uncertain |  |
| Jan-01-02 12:00:40 | 40 | Interpolated, Uncertain | Raw Value is Bad |
| Jan-01-02 12:00:45 | 45 | Interpolated, Uncertain | Bounding Value Bad |
| Jan-01-02 12:00:50 | 50 | Raw, Good |  |
| Jan-01-02 12:00:55 | 55 | Interpolated, Good |  |
| Jan-01-02 12:01:00 | 60 | Raw, Good |  |
| Jan-01-02 12:01:05 | 65 | Interpolated, Good |  |
| Jan-01-02 12:01:10 | 70 | Raw, Uncertain | Uncertain Values treated as Bad. |
| Jan-01-02 12:01:15 | 75 | Interpolated, Uncertain | Uncertain Values treated as Bad |
| Jan-01-02 12:01:20 | 80 | Raw, Good | Uncertain Values excluded. |
| Jan-01-02 12:01:25 | 85 | Interpolated, Good |  |
| Jan-01-02 12:01:30 | 90 | Raw, Good |  |

    - Interpolated data with good bounding value with bad data in the interval.

Start:  Jan-01-2002 12:00:35  End:  Jan-01-2002 12:01:00  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-02 12:00:35 | 35 | Interpolated, Uncertain |  |
| Jan-01-02 12:00:40 | 40 | Interpolated, Uncertain | Raw Value is Bad |
| Jan-01-02 12:00:45 | 45 | Interpolated, Uncertain | Bounding Value Bad |
| Jan-01-02 12:00:50 | 50 | Raw, Good |  |
| Jan-01-02 12:00:55 | 55 | Interpolated, Good |  |

    - Interpolated data with no good end bounding value.

Start:  Jan-01-2002 12:01:20  End:  Jan-01-2002 12:01:40  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |

    - Interpolated data with no good start bounding value.

Start:  Jan-01-2002 12:00:00  End:  Jan-01-2002 12:00:20  Interval: 00:00:05

| Historian 1 |  |  |  |
| --- | --- | --- | --- |
| Timestamp | Value | StatusCode | Notes |
| Jan-01-2002 12:00:00 | - | No Data, Bad | No bounding Value, do not extrapolate |
| Jan-01-2002 12:00:05 | - | No Data, Bad | No bounding value, do not extrapolate |
| Jan-01-2002 12:00:10 | 10 | Raw, Good |  |
| Jan-01-2002 12:00:15 | 15 | Interpolated, Good |  |